CPE301 – SPRING 2019

# Design Assignment 3B

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Xplained ATmega mini 328pb, jumper cables, LM35, FTDI Chip

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3/B

/*

* DA_3B_T1.c

*

* Created: 3/31/2019 9:35:23 AM

* Author : ERIC SMITH

The goal of the assignment is to modify the above codes to do the following:

1. Write a C AVR program that will monitor the LM34/35 connected to an Analog pin

(PC5) to display the temperature in F on the serial terminal every 1 sec. Use a timer

with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

2. Use the ATMEL Studio Data Visualizer or any Charting program to display the values

in time.

*/

#define F_CPU 16000000UL // 16 MHz

#define BAUDRATE 9600 // SET BAUD RATE 9600

#define BAUD_PRESCALLER ((((F_CPU / BAUDRATE * 16UL)))-1) // BAUD PRESCALER

// LIBRARY

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

void read_adc(void); // Function Declarations

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile unsigned int adc_temp;

char outs[20]; // ARRAY

int main(void) {

adc_init(); // INITIALIZE ADC )

USART_init(BAUD_PRESCALLER); // INITIALIZE USART

USART_tx_string("Connected!\r\n");

_delay_ms(125); // WAIT PERIOD

while(1) // WHILE LOOP

{

read_adc();

_delay_ms(1000); // WAIT PERIOD

}

}

void adc_init(void)

{

// SET UP AND ENABLE ADC

ADMUX = (0<<REFS1)| // REFERENCE SELECTION BITS

(1<<REFS0)| // AVcc - external cap at AREF

(0<<ADLAR)| // ADC Left Adjust

// SET FOR PC5

(1<<MUX2)| // ANALOG CHANNEL SELECTION BITS

(0<<MUX1)|

(1<<MUX0);

ADCSRA = (1<<ADEN)| // ADC ENABLE

(0<<ADSC)| // ADC Start Conversion

(0<<ADATE)| // ADC AUTO TRIGGER ENABLE

(0<<ADIF)| // ADC INTERRUPT FLAG

(0<<ADIE)| // ADC INTERRUPT ENABLE

(1<<ADPS2)| // ADC PRESCALER SELECT BITS

(0<<ADPS1)|

(1<<ADPS0);

// USING TIMER1

TIMSK1 = (1<<TOIE1); // ENABLE OVERFLOW INTERRUPT

TCCR1B |= (1<<CS11)|(1<<CS10); // PRESCALER

TCNT1 = 49911; // CALCULATED/NEEDED VALUE

}

/* READ ADC PINS */

// METHOD

void read_adc(void)

{

unsigned char i =4;

adc_temp = 0;

while (i--) {

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp / 4; // AVG. FEW SAMPLES

int tempf= (adc_temp*5*100)/1024;

snprintf(outs,sizeof(outs),"%3d\r\n", tempf); // KEY PRINT FUNCTION

USART_tx_string(outs);

}

void USART_init( unsigned int ubrr ) {

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); // ENABLE RECEIVER, TRANSMITTER & RX INTERRUPT

UCSR0C = (3 << UCSZ00); // ASYNCHRONOUS

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data ) {

while ((*data != '\0')) {

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

ISR(TIMER1_OVF_vect)

{

read_adc(); // ADC FUNCTION CALLED

snprintf(outs,sizeof(outs), "%3d\r\n", adc_temp); // KEY PRINT FUNCTION

USART_tx_string(outs); // STRING OUTPUT

_delay_ms(1000);

TCNT1 = 49911; // TIMER COUNTER RESET

}

- DEVELOPED MODIFIED CODE OF TASK 3B

/*

* DA_3B_T1.c

*

* Created: 3/31/2019 9:35:23 AM

* Author : ERIC SMITH

The goal of the assignment is to modify the above codes to do the following:

1. Write a C AVR program that will monitor the LM34/35 connected to an Analog pin

(PC5) to display the temperature in F on the serial terminal every 1 sec. Use a timer

with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

2. Use the ATMEL Studio Data Visualizer or any Charting program to display the values

in time.

*/

#define F_CPU 16000000UL // 16 MHz

#define BAUDRATE 9600 // SET BAUD RATE 9600

#define BAUD_PRESCALLER ((((F_CPU / BAUDRATE * 16UL)))-1) // BAUD PRESCALER

// LIBRARY

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

void read_adc(void); // Function Declarations

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile unsigned int adc_temp;

char outs[20]; // ARRAY

int main(void) {

adc_init(); // INITIALIZE ADC )

USART_init(BAUD_PRESCALLER); // INITIALIZE USART

USART_tx_string("Connected!\r\n");

_delay_ms(125); // WAIT PERIOD

while(1) // WHILE LOOP

{

read_adc();

_delay_ms(1000); // WAIT PERIOD

}

}

void adc_init(void)

{

// SET UP AND ENABLE ADC

ADMUX = (0<<REFS1)| // REFERENCE SELECTION BITS

(1<<REFS0)| // AVcc - external cap at AREF

(0<<ADLAR)| // ADC Left Adjust

// SET FOR PC5

(1<<MUX2)| // ANALOG CHANNEL SELECTION BITS

(0<<MUX1)|

(1<<MUX0);

ADCSRA = (1<<ADEN)| // ADC ENABLE

(0<<ADSC)| // ADC Start Conversion

(0<<ADATE)| // ADC AUTO TRIGGER ENABLE

(0<<ADIF)| // ADC INTERRUPT FLAG

(0<<ADIE)| // ADC INTERRUPT ENABLE

(1<<ADPS2)| // ADC PRESCALER SELECT BITS

(0<<ADPS1)|

(1<<ADPS0);

// USING TIMER1

TIMSK1 = (1<<TOIE1); // ENABLE OVERFLOW INTERRUPT

TCCR1B |= (1<<CS11)|(1<<CS10); // PRESCALER

TCNT1 = 49911; // CALCULATED/NEEDED VALUE

}

/* READ ADC PINS */

// METHOD

void read_adc(void)

{

unsigned char i =4;

adc_temp = 0;

while (i--) {

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp / 4; // AVG. FEW SAMPLES

int tempf= (adc_temp*5*100)/1024;

snprintf(outs,sizeof(outs),"%3d\r\n", tempf); // KEY PRINT FUNCTION

USART_tx_string(outs);

}

void USART_init( unsigned int ubrr ) {

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); // ENABLE RECEIVER, TRANSMITTER & RX INTERRUPT

UCSR0C = (3 << UCSZ00); // ASYNCHRONOUS

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data ) {

while ((*data != '\0')) {

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

ISR(TIMER1_OVF_vect)

{

read_adc(); // ADC FUNCTION CALLED

snprintf(outs,sizeof(outs), "%3d\r\n", adc_temp); // KEY PRINT FUNCTION

USART_tx_string(outs); // STRING OUTPUT

_delay_ms(1000);

TCNT1 = 49911; // TIMER COUNTER RESET

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)/MATERIALS NEEDED

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=1mTHR71gs1c&feature=youtu.be

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith