CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List: UNLV ECE BOARD, DC motor, male to male jumper wires, female to male jumper wires, L293D

- INITIAL/MODIFIED/DEVELOPED CODE OF Task 1 for DA4A

/*

* DA_4A.c

*

* Created: 4/13/2019 4:15:36 PM

* Author : Eric Smith

*/

#define F_CPU 16000000UL

// NEEDED LIBRARIES

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

// UART BAUDRATE

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

// DECLARATION OF KEY FUNCTIONS

void adc_init(void);

void read_adc(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

char outs[20];

volatile unsigned int adc_temp;

volatile unsigned int adc_value;

int main(void)

{

adc_init(); // ADC INITIALIZATION

USART_init(BAUD_PRESCALLER); // USART INITIALIZATION

USART_tx_string("Connected!\r\n");

_delay_ms(250); // WAIT A LITTLE BIT

// INITIALIZING PWM (PULSE WIDTH MODULATION)

DDRD = 0xFF;  // OUTPUT THE PWM, SET PORTD AS OUTPUT DIRECTION

TCCR2A = _BV(COM2A1) | _BV(COM2B1) | _BV(WGM21) | _BV(WGM20); // SET NEEDED BITS HIGH ( WGM21, WGM20, COM2A1, COM2B1 )

TCCR2B = _BV(WGM22) | _BV(CS20) | _BV(CS21) | _BV(CS22); // SET BIT WGM22 TO HIGH (FAST PWM MODE 7 RESET COUNTER) , SET CS20 TO HIGH

OCR2A = 255; // SET FOR WHEN COUNTER HITS 255, IT RESETS

OCR2B = 0; // REPRESENTS DUTY CYCLE

// PIN CHANGE INITILAIZER INTERRUPT

PCMSK1 |= (1 << PCINT10); // ENABLE PCINT10

PCICR |= (1 << PCIE1); // SET PIN CHANGE INTERRUPT, ENABLE REGISTER 1

sei(); // GLOBAL INTERRUPT SET

while (1) // WHILE LOOP NEEDED

{

read_adc();

_delay_ms(250

adc_value = adc_value / 4;

snprintf(outs,sizeof(outs),"%3d\r\n", adc_value);  // PRINT

USART_tx_string(outs); // ADC VALUE PRINTED

OCR2B = adc_value; // ADC VALUE REPLACED

}

}

void adc_init(void)

{

// SETUP & ENABLE ADC

ADMUX = (0<<REFS1)| // REFERENCE SELECTION BITS

(1<<REFS0)|    // AVcc - EXTERNAL CAP AT AREF

(0<<ADLAR)|    // ADC LEFT ADJUST RESULT

// ADJUST MUX

(0<<MUX2)|     // ANALOG CHANNEL SELECTION BITS

(0<<MUX1)|     // ADC0 - (PC0 PIN27)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENABLE

(0<<ADSC)|     // ADC START CONVERSION ADC

(0<<ADATE)|    // ADC AUTO TRIGGER ENABLE

(0<<ADIF)|     // ADC INTERRUPT FLAG

(0<<ADIE)|     // ADC INTERRUPT ENABLE

(1<<ADPS2)|    // ADC PRESCALER SELECT BITS

(0<<ADPS1)|

(1<<ADPS0);

}

void read_adc(void)

{

unsigned char i = 4;

adc_temp = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(100); // DELAY

}

adc_value = adc_value / 4;  // FEW SAMPLES SAMPLED

}

// SEND STRING TO RS-232

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

// INIT USART (RS-232)

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr >> 8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); // ENABLE RX, TX & RX INTERRUPT

UCSR0C = (3 << UCSZ00); // ASYNCHRONOUS 8 N 1

}

ISR(PCINT1_vect)

{

DDRD ^= 0xFF; // XOR DIRECTION REGISTER, TURN MKOTOR ON/OFF XOR

_delay_ms(1000); // BOUNCE GND

char check = " Pin Change ";

USART_tx_string(check); // PRINT AT SAME TIME OF PIN CHANGE

}

- DEVELOPED MODIFIED CODE OF TASK 1 for DA4A

(This portion covered by #2)

- SCHEMATICS

Key schematics:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO/UART PRINT ADC OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/VgW2OAu4ODM

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith