CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

UNLV ECE BOARD

Stepper Motor, wires, servo motor, UNL2003 motor driver

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1 (DA4B)

/*

* DA4B_T1.c

*

* Created: 4/21/2019 12:37:30 AM

* Author : ERIC SMITH

1. Write an AVR C program to control the speed of the Stepper Motor using a

potentiometer connected to PC0. Use a timer in CTC mode to control the delay.

*/

#define F_CPU 16000000UL

// NEEDED LIBRARIES

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <avr/interrupt.h>

void read_adc(void);

void adc_init(void);

volatile int OVFCOUNT;

volatile int RPMVALUE;

volatile double	float_value;

volatile unsigned int adc_value;

int start_value = 1699; // TIMER 1 INITIALIZED

float c = 1699; // OFFSETS AFFINE TRANSFORMATION

char outs[20]; // CHARACTER OUTS, 20

int main(void)

{

adc_init(); // ADC INITIALIZATION

_delay_ms(150); // WAIT A BIT

TCCR1B	|= (1<<WGM12)|(1<<CS11); // CTC MODE, PPRESCALER FOR 8

TIMSK1 |= (1<<OCIE1A); // MASK INTERRUPT

OCR1A = start_value; // 20,000us

DDRD = 0xFF; // PORTD AS OUTPUT

OVFCOUNT = 0; // GLOBAL OVERFLOW COUNTER SET TO 0

sei(); // GLOABL INTERRUPT

while (1)

{

// THIS LOOP CONSISTS OF AFFINE TRANSFORMATION

// KEY CALCULATION (PWM): (39999-1699) * adc_value / (1024-0) + 1699

read_adc();

float_value = adc_value;

RPMVALUE = (float_value * 37.4033) + c;

OCR1A = RPMVALUE; // NEWEST TIME MEASUREMENT LOADED

}

}

void adc_init(void)

{

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(0<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC0 (PC0 PIN23)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

void read_adc(void)

{

unsigned char i = 4;

adc_value = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_value+= ADC;

_delay_ms(100);

}

adc_value = adc_value / 4;  // AVG FEW SAMPLES

}

ISR(TIMER1_COMPA_vect)

{

OVFCOUNT++; // OVERFLOW FLAG COUNTER INCREMENT

int count; // LOCAL COUNTER INITIALIZED

count = OVFCOUNT; // GO HAND IN HAND, BOTH COUNT SIMILARLY

// 8 NECESSARY STEPS

if(count == 1){

PORTD = 0xE;

}

if(count == 2) {

PORTD = 0xC;

}

if(count == 3) {

PORTD = 0xD;

}

if(count == 4) {

PORTD = 0x9;

}

if(count == 5) {

PORTD = 0xB;

}

if(count == 6) {

PORTD = 0x3;

}

if(count == 7) {

PORTD = 0x7;

}

if(count == 8) {

PORTD = 0x6;

}

// COUNTER RESET IF COMPLETED

if(count == 8){

OVFCOUNT = 0; // STEPPER COUNTER RESTART

}

}

- DEVELOPED INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2 (DA4B)

/*

* DA4B_T2.c

*

* Created: 4/21/2019 1:49:04 AM

* Author : ERIC SMITH

2. Write an AVR C program to control the position of the Servo Motor using a

potentiometer connected to PC0. When pot value is 0 the servo is at position 0 deg.

and when pot value is max (approx. 5V) the servo is at position 180 deg.

*/

#define F_CPU 16000000UL

// NEEDED LIBRARIES

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void read_adc(void);

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile int PWMVALUE; // PWM VALUE

volatile unsigned int adc_value; // ADC VALUE

volatile double	float_value; // FLOAT VALUE

float c = 97; // AFFINE TRANSFORMATION, OFFSET NEEDED

char outs[20]; // CHARACTER OUTS, 20

int main(void)

{

TCCR1A |= (1 << COM1A1)|(1 << COM1B1)|(1 << WGM11); // TIMER 1 CONFIGURATED

TCCR1B|= (1 << WGM13)|(1 << WGM12)|(1 <<  CS11)|(1 << CS10); // PRESCALAR 64, WGM MODE 14, NON-INVERTED PWM

ICR1 = 4999; // KEY NUMBER

// SERVO MOTOR OPERATION STANDARD AT 20ms

DDRB |= (1<<PORTB1);

adc_init(); // ADC INITIALIZED

USART_init(BAUD_PRESCALLER);  // USART INITIALIZED

USART_tx_string("Connected!\r\n");

_delay_ms(150); // WAIT A BIT

while(1)

{

read_adc();

// KEY CALCULATION: adc_value * (535-97) / (1024 - 0) + 97

float_value = adc_value;

PWMVALUE = (float_value*0.4277343) + c;

snprintf(outs,sizeof(outs),"%3d\r\n", PWMVALUE);  // PRINT

USART_tx_string(outs);

OCR1A = PWMVALUE;

_delay_ms(150); // WAIT A BIT

}

}

void adc_init(void)

{

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(0<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC0 (PC0 PIN23)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

void read_adc(void)

{

unsigned char i = 4;

adc_value = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_value+= ADC;

_delay_ms(50);

}

adc_value = adc_value / 4;  // AVG FEW SAMPLES

}

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    // ASYNCHRONOUS 8 N 1

}

- SCHEMATICS

Key Schematics respectively:

- SCREENSHOTS OF EACH TASK OUTPUT

Task 1:

Task 2:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1:

Task 2:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/LF9_cKeLAUA

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith