CPE301 – SPRING 2019

# Design Assignment 6A

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List:

ATMega328p Xplained mini board

MPU6050 Board

Jumper cable wires

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA6A.c

*

* Created: 5/6/2019 8:08:36 PM

* Author : ERIC SMITH

Interface the provided MPU-6050 6-DOF IMU Sensor to the ATmega328p using the

I2C interface. Using the earlier developed code for UART, display the accelerometer

and gyro data to the UART Terminal. Extra credits for 1) visualizing the

accelerometer and gyro values (10 points), and 2) Apply Kalman Filtering on at least

one sensor data and display the filtered value.

*/

#define F_CPU 16000000UL

#ifndef F_CPU

#endif

// NEEDED LIBRARIES

#include <avr/io.h>

#include <util/delay.h>

#include <math.h>

#include <stdlib.h>

#include <stdio.h>

#include "MPUlibraries/MPU6050_def.h" // MPU 6050REGISTER DEFINE FILE

#include "MPUlibraries/i2c_master.h" // I2C MASTER HEADER FILE

#include "MPUlibraries/uart.h"	// USART HEADER FILE

#define MPU6050_WRITE 0xD0

#define MPU6050_READ 0xD1

// KEY FLOAT VARIABLES

float Acc_x;

float Acc_y;

float Acc_z;

float Gyro_x;

float Gyro_y;

float Gyro_z;

int main(void){

char buffer[20], float_[10];

char* PAX = "X Acceleration = ";

char* PAY = "Y Acceleration = ";

char* PAZ = "Z Acceleration = ";

char* PGX = "X Gyro Position = ";

char* PGY = "Y Gyro Position = ";

char* PGZ = "Z Gyro Position = ";

float Xa;

float Ya;

float Za;

float Xg;

float Yg;

float Zg;

init_uart(9600);

i2c_init();

init_MPU6050();

while(1){

getreading();

Xa = Acc_x/16384.0;	/* Divide raw value by sensitivity scale factor to get real values */

Ya = Acc_y/16384.0;

Za = Acc_z/16384.0;

Xg = Gyro_x/131;

Yg = Gyro_y/131;

Zg = Gyro_z/131;

dtostrf( Xa, 3, 2, float_ );/* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PAX);

USART_SendString(buffer);

_delay_ms(100);

dtostrf( Ya, 3, 2, float_ ); /* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PAY);

USART_SendString(buffer);

_delay_ms(100);

dtostrf( Za, 3, 2, float_ ); /* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PAZ);

USART_SendString(buffer);

USART_SendString("\r\n");

_delay_ms(1000);

dtostrf( Xg, 3, 2, float_ ); /* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PGX);

USART_SendString(buffer);

_delay_ms(100);

dtostrf( Yg, 3, 2, float_ ); /* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PGY);

USART_SendString(buffer);

_delay_ms(100);

dtostrf( Zg, 3, 2, float_ ); /* Take values in buffer to send all parameters over USART */

sprintf(buffer,"%s, ",float_);

USART_SendString(PGZ);

USART_SendString(buffer);

USART_SendString("\r\n");

_delay_ms(1000);

}

return 0;

}

void uart_putc(unsigned char c){

while(!(UCSR0A & (1<<UDRE0))); // WAIT A BIT, BEFORE SENDING CAN OCCUR

UDR0 = c; // OUTPUT CHARACTER, SAVED IN C

}

void uart_puts(char *s){

while(*s){

uart_putc(*s);

s++;

}

}

void init_uart(uint16_t baudrate){

uint16_t UBRR_val = (F_CPU/16)/(baudrate-1);

UBRR0H = UBRR_val >> 8;

UBRR0L = UBRR_val;

UCSR0B |= (1<<TXEN0) | (1<<RXEN0) | (1<<RXCIE0); // UART TX (TRANSMIT

UCSR0C |= (1<<USBS0) | (3<<UCSZ00); // MODUS ASYNCHRON

}

void init_MPU6050(void){

_delay_ms(150);

i2c_start(MPU6050_WRITE); // GYROSCOPE SAMPLE RATE = 1KHz, ACCELEROMETER SAMPLE RATE = 1KHz

i2c_write(SMPLRT_DIV); // SAMPLE RATE PRODUCED, DIVIFING GYROSCOPE OUTPUT RATE BY SMPLRT_DIV

i2c_write(0x07); // GYROSCOPE OUTPUT RATE = 8KHz, SAMPLE RATE = GYROSCOPE OUTPUT RATE / (1 + SMPLRT_DIV)

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(PWR_MGMT_1);

i2c_write(0x01); // X AXIS, PLL GYROSCOPE REFERENCE PLL

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(CONFIG); // FRAME SYNCHRONIZATION & DIGITAL LOW PASS FILTER

i2c_write(0x00);

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(GYRO_CONFIG); // GYROSCOPE SCALE RANGE = FS_SEL SELECTS = 11  = ± 2000 °/s

i2c_write(0x18);		// ACCELEROMETER RANGE  = ± 2g (default)

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(INT_ENABLE); // DATA_RDY_EN = 1

i2c_write(0x01);

i2c_stop();

}

void getreading(void){

i2c_start(MPU6050_WRITE);

i2c_write(ACCEL_XOUT_H); // POINTER SET

i2c_start(MPU6050_READ);

Acc_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

// ACCEL Y

Acc_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

// ACCEL Z

Acc_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

// GYRO X

Gyro_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

// GYRO Y

Gyro_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

// GYRO Z

Gyro_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

i2c_stop();

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

(NOT NEEDED, COVERED IN QUESTION #2)

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/P5H-sCUbQo8

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith