CPE301 – SPRING 2019

# Design Assignment 1B

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

N/A

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/B

Code:

;

; Lab1B-S19.asm

;

; Created: 2/23/2019 6:04:17 PM

; Author : Eric Smith

; Breakpoint used at RJMP END

; Chose consecutive numbers option instead of random numbers

; process of storing 99 numbers

RESET:

.SET STARTADDS = 0x0200

.SET COUNT = 99

.ORG 0

; POINTER SITUATION

LDI XL, LOW(STARTADDS) ; initializes [X] = 0x0200

LDI XH, HIGH(STARTADDS)

LDI YL, LOW(0x0400) ; initializes [Y] = 0x0400

LDI YH, HIGH(0x0400)

LDI ZL, LOW(0x0600) ; initializes [Z] = 0x0600

LDI ZH, HIGH(0x0600)

; COUNT REGISTER INITIALIZATION

LDI R21, COUNT ; R21 = 99

LDI R22, 10 ; R22 = 10 is starting point of counter

LDI R20, 3 ; R20 = 3

; CLEARING REGISTERS FOR SUMS

CLR R0

CLR R16

CLR R17

CLR R18

CLR R19

; ADDING LOWER ANDS HIGHER ADDRESS BITS TO BE STORED

START:

MOV R1, XL ; R1 = XL

ADD R1, XH ; R1 = XH + XL

MOV R3, R1 ; R3 = R1

ADD R3, R22 ; R3 = 10 + R3

ST X+, R3 ; stores values of R3 into incremented "X" register

DIVBYTHREE: ; divide by 3 loop

CP R1, R20 ; compare R20 to R1

BRLO DIVBAD ; if R1 < 3 go to DIVBAD

SUB R1, R20 ; R1 - 3

CP R1, R0 ; compare R0 to R1

BREQ DIVGOOD ; if R1 = 0 go to DIVGOOD

RJMP DIVBYTHREE

DIVGOOD: ; divide by 3 is good loop

SUB R3, R23 ; subtract R23 from R3

ST Y+, R3 ; stores value of R3 into incremented "Z" register

ADD R16, R3 ; R16 = ZH + ZL

ADC R17, R0 ; R17 = carry value from previous line

RJMP DONE

DIVBAD: ; divide by 3 is not good loop

SUB R3, R23 ; subtract R23 from R3

ST Z+, R3 ; stores value of R3 into incremented "Y" register

ADD R18, R3 ; R18 = YH + YL

ADC R19, R0 ; R19 = carry value from previous line

RJMP DONE

DONE:

CP R21, R0 ; compare R0 to R21

BREQ END ; if R21 = 0 go back to finish

SUBI R21, 1 ; R21 = 21 - 1

BRNE START ; if R21 != 0 go back to START

FINALVAL: ; final value part

ADD R3, R23 ; add R23 with r3

ADD R18, R3 ; add R3 with R18

ADC R19, R0 ; add with carry for R0 with R19

END:

RJMP END

- DEVELOPED MODIFIED CODE RESULTS

Thought process: If the numbers put in/used are consecutive rather than random, there will be the same final values in the registers at the end (despite the possible/multiple times of running the code)

Screenshot:

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

N/A

- VIDEO LINKS OF EACH DEMO

N/A

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT