CPE301 – SPRING 2019

# Design Assignment 2B

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components used were: UNLV ECE board, USB AVRISP XPII, and jumper cable.

- INITIAL/DEVELOPED/MODIFIED/ CODE OF TASK 2/B ASM CODE

; DA_2B_T1.asm

; Created: 3/9/2019 6:59:15 PM

; Author : Eric Smith

.include <m328pdef.inc>

.ORG 0x02 ; LOCATION FOR RESET

RJMP EXO_ISR

START:

LDI R16, 0x04 ; LOAD BINARY 0b00000100 INTO R16

OUT DDRB, R16 ; DIRECTION IS SET FOR R16 AS OUTPUT

OUT PORTB, R16 ; PULL-UP RESISTOR SET AT PIN3

LDI R17, 0x00 ; LOAD IMMEDIATE VALUE 0x00 INTO R17

OUT DDRD, R17 ; DIRECTION IS SET FOR R17 AS INPUT FOR ALL

SBI PORTD, 2 ; SET BIT, PULL-UP RESISTOR

NOP ; WAIT

; INTERRUPT INITIALIZATION

LDI R20, 0x2 ; MAKE INT0 FALLING EDGE TRIGGERED

STS EICRA, R20 ; CONFIGURE LOADED INTO R20

LDI R20, 1 << INT0 ; ENABLE INT0

OUT EIMSK, R20 ; (MASK) LOAD ENABLE INTO R20

SEI ; ENABLE GLOBAL INTERRUPTS

HERE: JMP HERE

EXO_ISR:

LDI R21, 0xFB ; LOAD BINARY 0b11111011 TO R21

OUT PORTB, R21 ; LED TURNED ON

RCALL DELAY_GO ; RUN THROUGH DELAY(S)

RCALL DELAY_100MS

RCALL DELAY_50MS

OUT PORTB, R16 ; LED TURNED OFF

RETI

; PROCESS OF USING DELAYS TO CALCULATE/GET 1.25 SECONDS

DELAY_GO:

RCALL DELAY_100MS ; 0.1 SECONDS

RCALL DELAY_100MS ; 0.2 SECONDS

RCALL DELAY_100MS ; 0.3 SECONDS

RCALL DELAY_100MS ; 0.4 SECONDS

RCALL DELAY_100MS ; 0.5 SECONDS

RCALL DELAY_100MS ; 0.6 SECONDS

RCALL DELAY_100MS ; 0.7 SECONDS

RCALL DELAY_100MS ; 0.8 SECONDS

RCALL DELAY_100MS ; 0.9 SECONDS

RCALL DELAY_100MS ; 1 SECOND

RCALL DELAY_100MS ; 1.1 SECONDS

RCALL DELAY_100MS ; 1.2 SECONDS

RET

DELAY_100MS:

RCALL DELAY_50MS ; 1.25 SECONDS

RET

DELAY_50MS:

PUSH R16

PUSH R18

LDI R16, 255 ; LOAD IMMEDIATE 255 TO R16

LDI R18, 6 ; LOAD IMMEDIATE 6 TO R18

DELAY_1A:

DELAY_1B:

NOP ; WAIT

DEC R16 ; DECREMENT R16

BRNE DELAY_1B

DEC R18 ; DECREMENT R18

BRNE DELAY_1B

POP R18 ; POP REGISTER FROM STACK

POP R16 ; POP REGISTER FROM STACK

RET

- INITIAL/DEVELOPED/MODIFIED CODE OF TASK 2/B C CODE

/*

* DA_2B_T2.c

* Created: 3/9/2019 3:38:11 PM

* Author : Eric Smith

Goal of assignment is to use GPIO Interrupts

Use INT0 (PD2 pin/LED3) Interrupt mechanism

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int main(void)

{

DDRB |= (1 << DDB2); // DIRECTION IS SET FOR PORTB2 AS OUTPUT

PORTB |= (1 << PORTB2); // PULL-UP RESISTOR IS TURNED ON FOR PORTB2

DDRD &= (0 << DDD2); // DIRECTION IS SET FOR PORTD2 AS INPUT

PORTD |= (1 << PORTD2); // PULL-UP RESISTOR IS TURNED ON FOR PORTD2

EICRA = 0x00; // INTERRUPT IS SET/ACTIVATED ON LOW LEVEL

EIMSK = (1 << INT0); // ENABLE EXTERNAL INTERRUPT

sei (); // ENABLES GLOBAL INTERRUPT

while (1); // WAIT HERE FOR INTERRUPT SITUATION

}

ISR (INT0_vect) // ISR FOR EXTERNAL INTERRUPT

{

PORTB &= (0 << PORTB2); // LED IS TURNED ON

_delay_ms(1250); // 1.25 SECOND DELAY (SAME AS DESIGN ASSIGNMENT 2A TIME DELAY)

PORTB |= (1 << PORTB2); // LED IS TURNED ON AFTER MANUAL INTERRUPT

}

- SCHEMATICS

Key Schematic:

Key Diagram:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Assembly Code Atmel Studio Output:

C Code Atmel Studio Output:

- SCREENSHOT OF DEMO (BOARD SETUP)

- VIDEO LINK OF DEMONSTRATION

https://youtu.be/309Nv3_379g

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith