CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components used were: UNLV ECE board, USB AVRISP XPII, and jumper cables

- INITIAL DEVELOPED C CODE OF TASK 3/A

GOAL OF ASSIGNMENT: to modify the above codes to do the following: 1. Write a C AVR program that will display a string, random integer and floating point values on the serial terminal every 1 sec. Use a timer with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

/*

* DA_3A_T1.c

*

* Created: 3/27/2019 6:25:30 PM

* Author : ERIC SMITH

*/

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

volatile int Count;

int n;

float PI;

char outs[30];

char str[] = "UNLV IS THE BEST";

char empty[] = " ";

void USART_init( unsigned int ubrr );

void USART_TX_string(char *data);

int main(void)

{

Count = 0;

USART_init(BAUD_PRESCALLER);

USART_TX_string("Connected!\r\n");

TIMSK0 |= (1 << TOIE0);

sei();

TCCR0A = 0x00;

TCCR0B = 0x05;

while(1);

}

void USART_TX_string(char *data)

{

while (*data != '\0') {

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;

data++;

}

}

void USART_init(unsigned int ubrr)

{

UBRR0H = (unsigned char)(ubrr >> 8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

ISR (TIMER0_OVF_vect)

{

while (Count < 61)

{

if ((TIFR0 & 0x01)  == 1)

{

TIFR0 = 0x01;

Count++;

}

}

if (Count > 60)

{

USART_TX_string(str);

USART_TX_string(empty);

n = rand();

PI = 3.1415;

dtostrf(PI, 6, 6, outs);

USART_TX_string(outs);

USART_TX_string(empty);

snprintf(outs, sizeof(outs), "%5d\r\n");

USART_TX_string(outs);

USART_TX_string(empty);

USART_TX_string(str);

USART_TX_string(empty);

Count = 0;

}

}

- DEVELOPED MODIFIED/COMMENTED C CODE OF DA 3A from TASK

/*

* DA_3A_T1.c

*

* Created: 3/27/2019 6:25:30 PM

* Author : ERIC SMITH

*/

#define F_CPU 8000000UL // 8MHz

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

#define BAUDRATE 9600 // DEFINITION REGARDING BAUDRATE

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1) // CONFIRMATION OF BAUDRATE PRESCALER

volatile int Count; // ESTABLISHMENT OF INTEGER COUNT NEEDED

int n; // VARIABLE N INTEGER

float PI; // FLOAT PI REPRESENTATION

char outs[30]; // CHARACTER ARRAY USED TO STORE RESPECTIVE AMOUNT

char str[] = "UNLV IS THE BEST"; // STRING TO BE DISPLAYED

char empty[] = " "; // EMPTY STRING FOR SPACE

void USART_init( unsigned int ubrr ); // VOID FUNCTION

void USART_TX_string(char *data); // VOID FUNCTION

int main(void)

{

Count = 0; // COUNT SET AT ZERO

USART_init(BAUD_PRESCALLER); // FOR USART, SETS BAUD PRESCALER

USART_TX_string("Connected!\r\n"); // FOR USART, SETS TEXT STRING

TIMSK0 |= (1 << TOIE0); // SETS INTERRUPT ACTIVATION

sei(); // ENABLES GLOBAL INTERRUPT

TCCR0A = 0x00; // NORMAL MODE REGARDING TIMER

TCCR0B = 0x05;

while(1);

}

void USART_TX_string(char *data) // VOID FUNCTION FOR STRING

{

while (*data != '\0') {

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;

data++;

}

}

void USART_init(unsigned int ubrr // VOID FUNCTION FOR USART_INIT

{

UBRR0H = (unsigned char)(ubrr >> 8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); // INTERRUPT FOR TX ENABLED

UCSR0C = (3 << UCSZ00); // ASYNCHRONOUS

}

ISR (TIMER0_OVF_vect) // FLAG INTERRUPT FOR TV0V

{

while (Count < 61) // LOOP WHEN LESS THAN 61

{

if ((TIFR0 & 0x01)  == 1)

{

TIFR0 = 0x01;

Count++;

}

}

if (Count > 60) // PERFORM/EXECUTE WHEN COUNT GREATER THAN 60

{

USART_TX_string(str); // PRINT STRING

USART_TX_string(empty); // BLANK SPACE PRINTED

n = rand(); // RANDOM NUMBER GENERATED

PI = 3.1415; // CHOSE PI

dtostrf(PI, 6, 6, outs); // DOUBLE VALUE PASSED IS CONVERTED IN VAL INTO ASCII REPRESENTATION TO BE STORED UNDER S

USART_TX_string(outs);

USART_TX_string(empty);

snprintf(outs, sizeof(outs), "%5d\r\n"); // CONVERTS FLOATING VALUE

USART_TX_string(outs); // WRITES STRING

USART_TX_string(empty);

USART_TX_string(str);

USART_TX_string(empty); // BLANK SPACE TO SERIAL

Count = 0; // RESETS COUNTER

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=NZbKV6-F2Ow&feature=youtu.be

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith