CPE301 – SPRING 2019

# Design Assignment 5A

Student Name: Eric Smith

Student #: 2000543940

Student Email: smithe4@unlv.nevada.edu

Primary Github address: smitheas95

Directory: Repository

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List:

Nrf24L01

ATMega328p

LM34

Male to female jumper wires

Pin Out:

- INITIAL/MODIFIED/DEVELOPED CODE OF DA5A

Main Code:

/*

* DA_5A.c

*

* Created: 4/27/2019 9:19:57 PM

* Author : Eric Smith

Interface the provided NRF24L01+ RF Module to the ATmega328p using the SPI

interface. Using the earlier developed code for ADC, transmit the ADC value of the

internal temperature sensor, or LM35 sensor between two RF Modules. The

ATmega328p interfacing the RF Modules should alternate between TX and RX modes

every 0.5 secs (hopefully they are not both at TX and RX modes in the same interval).

The temperature of both ATmega328p’s should be displayed on both ATmega328p’s.

*/

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

// NEDDED LIBRARIES

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

// SET UP UART FOR PRINTF();

#ifndef BAUD

#define BAUD 9600

#endif

#include "inctest/STDIO_UART.h"

// NRF24L01+ LIBRARY INCLUDED

#include "inctest/nrf24l01.h"

#include "inctest/nrf24l01-mnemonics.h"

#include "inctest/spi.h"

// IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

// CLICHE MESSAGE TO SEND

char tx_message[32]; // ARRAY STRING DEFINED

int main(void)

{

strcpy(tx_message, " Hi, This message is from Eric Smith!"); // COPY STRING INTO ARRAY

// UART INITIALIZED

uart_init();

// Initialize nRF24L01+ and print configuration info

nrf24_init();

// PRINT_CONFIGU();

// BEGIN LISTENING TO INCOMING MESSAGES

nrf24_start_listening();

status = nrf24_send_message(tx_message);

if (status == true)

{

printf("Connection Established\n");

}

while(1)

{

if(message_received)

{

// MESSAGE TO BE RECEIVED, PRINT IT

message_received = false; // Reset to false

printf("Received Temperature: %s\n", nrf24_read_message());

// RESPONSE AS A MESSAGE

_delay_ms(500);

// PERFORM ADC EXECUTION, STORE IN TX_MESSAGE

status = nrf24_send_message(tx_message);

if(status == true)

{

printf("Temperature Transmitted\n");

}

}

}

}

// IRQ pin INTERRUPT

ISR(INT0_vect)

{

message_received = true;

}

Nrf24L01 Code:

// Set clock frequency

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdbool.h>

#include <string.h>

// nRF24L01+ include files

#include "nrf24l01.h"

#include "nrf24l01-mnemonics.h"

#include "spi.h"

// Settings

uint8_t rx_address[5] = { 0x55, 0x55, 0x55, 0x55, 0x55 };	// Read pipe address

uint8_t tx_address[5] = { 0x66, 0x66, 0x66, 0x66, 0x66 };	// Write pipe address

#define READ_PIPE		0									// Number of read pipe

//

// -AUTO_ACK can be disabled when running on 2MBPS @ <= 32 byte messages.

// -250KBPS and 1MBPS with AUTO_ACK disabled lost many packets

// if the packet size was bigger than 4 bytes.

// -If AUTO_ACK is enabled, tx_address = rx_address.

//

#define AUTO_ACK		false								// Auto acknowledgment

#define DATARATE		RF_DR_2MBPS							// 250kbps, 1mbps, 2mbps

#define POWER			POWER_MAX							// Set power (MAX 0dBm..HIGH -6dBm..LOW -12dBm.. MIN -18dBm)

#define CHANNEL			0x74								// 2.4GHz-2.5GHz channel selection (0x01 - 0x7C)

#define DYN_PAYLOAD		true								// Dynamic payload enabled

#define CONTINUOUS		false								// Continuous carrier transmit mode (not tested)

//

// ISR(INT0_vect) is triggered depending on config (only one can be true)

//

#define RX_INTERRUPT	true								// Interrupt when message is received (RX)

#define TX_INTERRUPT	false								// Interrupt when message is sent (TX)

#define RT_INTERRUPT	false								// Interrupt when maximum re-transmits are reached (MAX_RT)

//

// -PIN map.

// -If CE or CSN is changed to different PIN e.g. PC0

// then change DDRB -> DDRC, PORTB -> PORTC and so on

//

// CE

#define CE_DDR		DDRB

#define CE_PORT		PORTB

#define CE_PIN		DDB1									// CE connected to PB1

// CSN

#define CSN_DDR		DDRB

#define CSN_PORT	PORTB

#define CSN_PIN		DDB2									// CSN connected to PB2

// IRQ

#define IRQ_DDR		DDRD

#define IRQ_PORT	PORTD

#define IRQ_PIN		DDD2									// IRQ connected to PD2

// MOSI

#define MOSI_DDR	DDRB

#define MOSI_PORT	PORTB

#define MOSI_PIN	DDB3

// MISO

#define MISO_DDR	DDRB

#define MISO_PORT	PORTB

#define MISO_PIN	DDB4

// SCK

#define SCK_DDR		DDRB

#define SCK_PORT	PORTB

#define SCK_PIN		DDB5

// PIN toggling

#define setbit(port, bit) (port) |= (1 << (bit))

#define clearbit(port, bit) (port) &= ~(1 << (bit))

#define ce_low clearbit(CE_PORT,CE_PIN)

#define ce_high setbit(CE_PORT,CE_PIN)

#define csn_low clearbit(CSN_PORT,CSN_PIN)

#define csn_high setbit(CSN_PORT,CSN_PIN)

// Used to store SPI commands

uint8_t data;

uint8_t nrf24_send_spi(uint8_t register_address, void *data, unsigned int bytes)

{

uint8_t status;

csn_low;

status = spi_exchange(register_address);

for (unsigned int i = 0; i < bytes; i++)

((uint8_t*)data)[i] = spi_exchange(((uint8_t*)data)[i]);

csn_high;

return status;

}

uint8_t nrf24_write(uint8_t register_address, uint8_t *data, unsigned int bytes)

{

return nrf24_send_spi(W_REGISTER | register_address, data, bytes);

}

uint8_t nrf24_read(uint8_t register_address, uint8_t *data, unsigned int bytes)

{

return nrf24_send_spi(R_REGISTER | register_address, data, bytes);

}

void nrf24_init(void)

{

// Interrupt on falling edge of INT0 (PD2) from IRQ pin

cli();					// Disable interrupts

EICRA |= (1 << ISC01);

EIMSK |= (1 << INT0);

sei();					// Enable interrupts

// CSN and CE as outputs and initial states

setbit(CE_DDR,CE_PIN);

setbit(CSN_DDR,CSN_PIN);

csn_high;

ce_low;

// Initialize SPI

spi_master_init();

_delay_ms(100);				// Power on reset 100ms

// Start nRF24L01+ config

data =

(!(RX_INTERRUPT) << MASK_RX_DR) |	// IRQ interrupt on RX (0 = enabled)

(!(TX_INTERRUPT) << MASK_TX_DS) |	// IRQ interrupt on TX (0 = enabled)

(!(RT_INTERRUPT) << MASK_MAX_RT) |	// IRQ interrupt on auto retransmit counter overflow (0 = enabled)

(1 << EN_CRC) |						// CRC enable

(1 << CRC0) |						// CRC scheme

(1 << PWR_UP) |						// Power up

(1 << PRIM_RX);						// TX/RX select

nrf24_write(CONFIG,&data,1);

// Auto-acknowledge on all pipes

data =

(AUTO_ACK << ENAA_P5) |

(AUTO_ACK << ENAA_P4) |

(AUTO_ACK << ENAA_P3) |

(AUTO_ACK << ENAA_P2) |

(AUTO_ACK << ENAA_P1) |

(AUTO_ACK << ENAA_P0);

nrf24_write(EN_AA,&data,1);

// Set retries

data = 0xF0;				// Delay 4000us with 1 re-try (will be added in settings)

nrf24_write(SETUP_RETR,&data,1);

// Disable RX addresses

data = 0;

nrf24_write(EN_RXADDR, &data, 1);

// Set channel

data = CHANNEL;

nrf24_write(RF_CH,&data,1);

// Setup

data =

(CONTINUOUS << CONT_WAVE) |					// Continuous carrier transmit

((DATARATE >> RF_DR_HIGH) << RF_DR_HIGH) |	// Data rate

((POWER >> RF_PWR) << RF_PWR);				// PA level

nrf24_write(RF_SETUP,&data,1);

// Status - clear TX/RX FIFO's and MAX_RT by writing 1 into them

data =

(1 << RX_DR) |								// RX FIFO

(1 << TX_DS) |								// TX FIFO

(1 << MAX_RT);								// MAX RT

nrf24_write(STATUS,&data,1);

// Dynamic payload on all pipes

data =

(DYN_PAYLOAD << DPL_P0) |

(DYN_PAYLOAD << DPL_P1) |

(DYN_PAYLOAD << DPL_P2) |

(DYN_PAYLOAD << DPL_P3) |

(DYN_PAYLOAD << DPL_P4) |

(DYN_PAYLOAD << DPL_P5);

nrf24_write(DYNPD, &data,1);

// Enable dynamic payload

data =

(DYN_PAYLOAD << EN_DPL) |

(AUTO_ACK << EN_ACK_PAY) |

(AUTO_ACK << EN_DYN_ACK);

nrf24_write(FEATURE,&data,1);

// Flush TX/RX

// Clear RX FIFO which will reset interrupt

uint8_t data = (1 << RX_DR) | (1 << TX_DS) | (1 << MAX_RT);

nrf24_write(FLUSH_RX,0,0);

nrf24_write(FLUSH_TX,0,0);

// Open pipes

nrf24_write(RX_ADDR_P0 + READ_PIPE,rx_address,5);

nrf24_write(TX_ADDR,tx_address,5);

nrf24_write(EN_RXADDR,&data,1);

data |= (1 << READ_PIPE);

nrf24_write(EN_RXADDR,&data,1);

}

void nrf24_write_ack(void)

{

const void *ack = "A";

unsigned int length = 1;

csn_low;

spi_send(W_ACK_PAYLOAD);

while (length--) spi_send(*(uint8_t *)ack++);

csn_high;

}

void nrf24_state(uint8_t state)

{

uint8_t config_register;

nrf24_read(CONFIG,&config_register,1);

switch (state)

{

case POWERUP:

// Check if already powered up

if (!(config_register & (1 << PWR_UP)))

{

data = config_register | (1 << PWR_UP);

nrf24_write(CONFIG,&data,1);

// 1.5ms from POWERDOWN to start up

_delay_ms(2);

}

break;

case POWERDOWN:

data = config_register & ~(1 << PWR_UP);

nrf24_write(CONFIG,&data,1);

break;

case RECEIVE:

data = config_register | (1 << PRIM_RX);

nrf24_write(CONFIG,&data,1);

// Clear STATUS register

data = (1 << RX_DR) | (1 << TX_DS) | (1 << MAX_RT);

nrf24_write(STATUS,&data,1);

break;

case TRANSMIT:

data = config_register & ~(1 << PRIM_RX);

nrf24_write(CONFIG,&data,1);

break;

case STANDBY1:

ce_low;

break;

case STANDBY2:

data = config_register & ~(1 << PRIM_RX);

nrf24_write(CONFIG,&data,1);

ce_high;

_delay_us(150);

break;

}

}

void nrf24_start_listening(void)

{

nrf24_state(RECEIVE);				// Receive mode

//if (AUTO_ACK) nrf24_write_ack();	// Write acknowledgment

ce_high;

_delay_us(150);						// Settling time

}

uint8_t nrf24_send_message(const void *tx_message)

{

// For printf();

char temp[32];

memset(temp,0,32);

strcpy(temp,tx_message);

// Message length

uint8_t length = strlen(tx_message);

// Transmit mode

nrf24_state(TRANSMIT);

// Flush TX/RX and clear TX interrupt

nrf24_write(FLUSH_RX,0,0);

nrf24_write(FLUSH_TX,0,0);

data = (1 << TX_DS);

nrf24_write(STATUS,&data,1);

// Disable interrupt on RX

nrf24_read(CONFIG,&data,1);

data |= (1 << MASK_RX_DR);

nrf24_write(CONFIG,&data,1);

// Start SPI, load message into TX_PAYLOAD

csn_low;

if (AUTO_ACK) spi_send(W_TX_PAYLOAD);

else spi_send(W_TX_PAYLOAD_NOACK);

while (length--) spi_send(*(uint8_t *)tx_message++);

spi_send(0);

csn_high;

// Send message by pulling CE high for more than 10us

ce_high;

_delay_us(15);

ce_low;

// Wait for message to be sent (TX_DS flag raised)

nrf24_read(STATUS,&data,1);

while(!(data & (1 << TX_DS))) nrf24_read(STATUS,&data,1);

printf("Message sent: %s\n",temp);

// Enable interrupt on RX

nrf24_read(CONFIG,&data,1);

data &= ~(1 << MASK_RX_DR);

nrf24_write(CONFIG,&data,1);

// Continue listening

nrf24_start_listening();

return 1;

}

unsigned int nrf24_available(void)

{

uint8_t config_register;

nrf24_read(FIFO_STATUS,&config_register,1);

if (!(config_register & (1 << RX_EMPTY))) return 1;

return 0;

}

const char * nrf24_read_message(void)

{

// Message placeholder

static char rx_message[32];

memset(rx_message,0,32);

// Write ACK message

if (AUTO_ACK) nrf24_write_ack();

// Get length of incoming message

nrf24_read(R_RX_PL_WID,&data,1);

// Read message

if (data > 0) nrf24_send_spi(R_RX_PAYLOAD,&rx_message,data+1);

// Check if there is message in array

if (strlen(rx_message) > 0)

{

// Clear RX interrupt

data = (1 << RX_DR);

nrf24_write(STATUS,&data,1);

return rx_message;

}

// Clear RX interrupt

data = (1 << RX_DR);

nrf24_write(STATUS,&data,1);

return "failed";

}

- DEVELOPED MODIFIED CODE OF DA5A

(Not needed, covered in question 2)

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Sending and Receiving Outputs Via Data Visualizer:

NOTE: My partner was Matt Tomljenovic. We adjusted our address values to make for successful completion of the sending and receiving process.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/-_NiYpt7hv8

- GITHUB LINK OF THIS DA

https://github.com/smitheas95/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Eric Smith